Event-based server implementation in Linux

Description

Event-based implementations of servers are more robust than their thread-based counterpart since the number of working threads is bounded and predetermined. However, the use of asynchronous system calls complicates the development and may end up adding a significant overhead.

In this project you should define a server that provides a service that involves both CPU (e.g., some arithmetic operations in a loop) and I/O (e.g., access to a large file). Then, start with an iterative approach, then a thread-based approach and finally an event-based approach. Finally, compare the overhead and performance (e.g., throughput and latency) of all the three approaches when you increase the number of concurrent clients.

References

Matt Welsh, David Culler, and Eric Brewer. 2001. SEDA: an architecture for well-conditioned, scalable Internet services. SIGOPS Oper. Syst. Rev. 35,5 (Dec.2001), 230243. https://doi.org/10.1145/502059.502057

PTP client-side clock synchronization with servo-clock

Description

PTP is a synchronization protocol that allows achieving higher accuracy than NTP (in principle). The idea is to compare both protocols using standard servers (PTP and NTP) and clients that synchronize their clocks with a servo-clock approach.

In this project, find PTP and NTP servers and install them on a computer in a LAN. Then Implement the PTP and NTP clients with servo-clocks. Measure the synchronization error and how it varies with relevant control parameters of the servo-clock in both cases.

References

André Perez, Synchronization IEEE 1588 Mechanism, in Ed. André Perez, Implementing IP and Ethernet on the 4G Mobile Network, Elsevier, 2017, Pages 253-267, ISBN 9781785482380, https://doi.org/10.1016/B978-1-78548-238-0.50015-7.

Miroslav Lichvar, Combining PTP with NTP to Get the Best of Both Worlds, Red Hat Blog, July 20, 2016. https://www.redhat.com/en/blog/combining-ptp-ntp-get-best-both-worlds

Experimental overhead comparison of CoAP and MQTT in embedded nodes

Description

CoAP and MQTT are standard application layer protocols widely used in the IoT, particularly for low resource nodes. In these nodes it is important to assess and consider the overhead imposed by these protocols.

In this project, implement an asynchronous group communication with CoAP described in the following paper and then compare with MQTT. Use a network of ESP32 or ESP8266 nodes.

References

J. Misic, M. Z. Ali and V. B. Misic, "Protocol Architectures for IoT Domains," in IEEE Network, vol. 32, no. 4, pp. 81-87, July/August 2018, doi: 10.1109/MNET.2018.1700395.

Application-Level Multicast with the Banana-Tree Protocol

Description

Many distributed applications use group communication. In these cases, it is frequently needed to carry out multicasting over a global network that supports unicasting, only, or supports multicasting in very limited ways (e.g., the Internet). Therefore, it is relevant to develop an overlay network that offers the desired multicasting services.

In this project you will develop an overlay multicast network over the Internet using a tree approach, particularly the Banana-Tree Protocol

References

Helder, D.A., & Jamin, S., Banana Tree Protocol, an End-host Multicast Protocol.

Experimental characterization of the DDS middleware

Description

Publisher-Subscriber middleware became very common in a myriad of application domains where multiple users can subscribe to the same data items. One of the first such middlewares that considered multiple non-functional properties, particularly timeliness, was DDS – Data Distribution Service.

In this project install DDS and use it to implement a Real-Time Database in a distributed shared memory style. Then assess multiple properties of interest, namely freshness of data and latency of its use.

References

Use the DDS version of RTI https://www.rti.com/products/dds-standard . If problems arise, then use OpenDDS.

DDS Tutorial: Angelo Corsaro. https://pt.scribd.com/document/486008534/SLides-A-Corsaro-pdf

Gossip vs anti-entropy for epidemic information dissemination

Description

In certain applications it is desired to spread information among a large number of nodes in an organic fashion, possibly connected to a global network that supports essentially unicast communication (e.g., Internet). One possibility is to use an epidemic approach.

In this project, generate a large network of nodes with random topology and then implement both gossip and anti-entropy epidemic dissemination protocols. Compare the time to reach all nodes, the probability to stop before covering all nodes and the number of messages exchanged.

References

Alberto Montresor. Gossip and Epidemic Protocols. http://disi.unitn.it/~montreso/ds/papers/montresor17.pdf